Background

CSD0201 was drafted in early summer 2007; little detail was provided about the format of the Settlement reports. The CMA has developed this guidance note as interim measures to assist Trading Parties understand how report is derived. This guidance note will be subsumed into CSD0201 in due course.

Structure of this Guidance Note

- Section 1 defines the Comma Separated Variable record layout by the use of an example report;

- Sections 2 defines the various header and trailer records contained within a file; and

- Sections 3-7 provide a description of the data and its numerical derivation.

Section 1 – Record Layout

The output file is a CSV with four columns and is structured as follows:

- Each file has 5 header rows shown in the example below as H1=>H5. The date format in H4 is as shown in the example;

- Each License Provider commences with three header rows shown in the example below as LPH1=>LPH3;

- Water and Sewerage charges are shown in repeating blocks. SE1, SE2 and SE3 are always at the start of each block. Service Elements are listed in rows SE4a=>Se4z. These rows are data driven for instance if there a no 15mm meter then there is no 15mm meter row. The block ends with a sub-total line SE5;

- Trade Effluent is shown in a unique block. TE1, TE2 and TE3 are always at the start of the block. Each DPID is specifically listed in rows TE4a=>Te4z. These rows are data driven for to provide a row for each DPID. The block ends with a sub-total line TE5;

- Each License Provider ends with two trailer rows shown in the example below as LPT1 and LPTH2.

For the avoidance of doubt column 4 for the row LPH4 shows the volume however the detail rows SE4x and TE4x is show the charges in pence.

| Row/Col |  | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| H1 | Type: | RUN_ONE |  |  |
| H2 | Tariff Year: | 2008 |  |  |
| H3 | Invoice Period: | 2: 01/05/2008 - 31/05/2008 |  |  |
| H4 | Scheduled Run Date: | 39600 |  |  |
| H5 |  |  |  |  |
| LPH1 | LP: | WorldWideWaterCompany |  |  |
| LPH2 |  |  |  |  |
| LPH3 | Total Charge= | 84083950 | Total Volume= | 536320.3424 |
| SE1 |  |  |  |  |
| SE2 | Water Volumetric Charges |  |  |  |
| SE3 | Service Element | Number of registered days | Volume / m3 | Charge / pence |
| SE4a | 15mm | 146 | 730.85 | 50383.38 |
| SE4b | 20mm | 202 | 402.8 | 14708.42 |
|  | 25mm | 168 | 1345.67 | 102375.52 |
|  | 50mm | 31 | 4506.8 | 317357.91 |
|  | 200mm | 62 | 49662.45 | 8038889.66 |
|  | 250mm | 62 | 556.3 | 124309.43 |
| SE4z | 20mm (unmeasurable) | 365 | 1348.162 | 74720.44 |
| SE5 | Sub Total |  | 58553.03 | 8722744.76 |
| SE1 |  |  |  |  |
| SE2 | Water Non Volumetric Charges |  |  |  |
| SE3 | Service Element | Number of registered days |  | Charge / pence |
| SE4a | 15mm | 146 |  | 1541.92 |
| SE4b | 20mm | 202 |  | 513.97 |
|  | 25mm | 168 |  | 5937.53 |
|  | 50mm | 31 |  | 7822.19 |
|  | 200mm | 279 |  | 12084015.34 |
|  | 250mm | 93 |  | 9084015.34 |
| SE4z | 20mm (unmeasurable) | 365 |  | 5011.23 |
| SE5 | Sub Total |  |  | 21188857.52 |
| SE1 |  |  |  |  |
| SE2 | Sewerage Volumetric Charges |  |  |  |
| SE3 | Service Element | Number of registered days | Volume / m3 | Charge / pence |
| SE4a | 15mm | 87 | 346.97 | 42578.73 |
| SE4b | 20mm | 27 | 189.37 | 23260.5 |
|  | 25mm | 81 | 520.49 | 68491.91 |
|  | 50mm | 30 | 250.19 | 42168.89 |
|  | 200mm | 2604 | 267880.5 | 36862383.79 |
|  | 250mm | 806 | 204599.7 | 16109522.68 |
| SE4z | 20mm (unmeasurable) | 254 | 947.1009 | 76598.79 |
| SE5 | Sub Total |  | 474734.3 | 53225005.29 |
| SE1 |  |  |  |  |
| SE2 | Sewerage Non Volumetric Charges |  |  |  |
| SE3 | Service Element | Number of registered days |  | Charge / pence |
| SE4a | 15mm | 87 |  | 369.59 |
| SE4b | 20mm | 27 |  | 369.59 |
|  | 25mm | 81 |  | 3395.34 |
|  | 50mm | 30 |  | 7397.26 |
|  | 200mm | 31 |  | 117783.01 |
|  | 250mm | 62 |  | 535566.03 |
|  | 20mm (unmeasurable) | 254 |  | 3417.53 |
|  | Road Drainage | 582 |  | 202264.39 |
| SE4z | Property Drainage RV | 582 |  | 76717.75 |
| SE5 | Sub Total |  |  | 947280.49 |
| TEH1 |  |  |  |  |
| TEH2 | Trade Effluent Charges |  |  |  |
| TEH3 | Service Element | Number of registered days | Volume / m3 | Charge / pence |
| TE4a | Trade Effluent x1234 | 30 | 1354.39 | 777115.37 |
| TE4b | Trade Effluent 783A01 | 31 | 451.88 | 19807.89 |
|  | Trade Effluent 0005A02 | 31 | 466.15 | 18688.29 |
| TE4z | Trade Effluent 0003B01 | 31 | 760.55 | 85176.77 |
| TE5 | Sub Total |  | 3032.97 | 900788.32 |
| LPT1 |  |  |  |  |
| LPT2 | END LP: | WorldWideWaterCompany |  |  |

Section 2 - Header and Generic Data Items

| Report Header |  |
| --- | --- |
| Type: | States which Settlement Run Type has been performed, as defined in CSD0201. |
| Tariff Year: | States the Financial Year which the Settlement Report belong to quoting the calendar year in which it begins. |
| Invoice Period (IP): | Provides the numerical value and date range that apply for that IP. This is a calendar month. |
| Scheduled Run Date: | The date that the Settlement Report was produced. This occurs at the beginning of the clock day (i.e. occurs before the start of the Business day if run on a Business Day). |
| Gap Line: | Provided before the first LP |
| Header for Each LP |  |
| LP: | The name of the Licensed Provider who’s Charges follow. |
| Gap Line: | Provided after the LP name |
| LPH3 Total Charge | The total charge for the LP named in the header. Includes all Service Elements relating to Water, Sewerage and Trade Effluent. |
| LPH3Total Volume | The total volume associated to both Water and Sewerage Service Volumetric Charges. It excludes Trade Effluent Volume |
| Trailer for each LP |  |
| Gap Line: | Provided before the “End LP” |
| End LP | The name of the LP that has been reported in the detailed lines above. |

Section 3 - Water Volumetric Charges

These exclude the fixed charge based elements of meter based charges. SPIDs subject to Transitional charges will appear twice; once for the physical based Chargeable Meter Size (CMS) and once for the Rateable Value based CMS.

| Service Elements | The column labelled Service Element provides the level of Reporting in Aggregate for each “Chargeable Meter Size” (CMS) that has been notified or derived. Meter and Unmeasureable CMS are reported separately. Unmeasureable CMS includes all RV derived Supply Points (i.e. Unmeasureable and Transition Arrangement SPIDs). |
| --- | --- |
| Number of registered days | The Number of registered days describes how many days the LP has been registered to each SPID that is aggregated for the Service Element being reported. This value is unaffected by the Vacancy or Temporary Disconnected status and is unaffected by the Transition percentage is being applied. There are two counts for Transition SPIDs with meters, the physical based CMS and the RV based CMS (reported as an Unmeasureable Service Element e.g. 20mm Unmeasurable. |
| Volume / m3 | The volume reported is the volume associated to either the meter or the RV (for Unmeasureable CMS). It is the sum of Daily Volumes for each day the LP is registered to each SPID summed up for that Service Element CMS. The volume figure is calculated as follows: - Transition Volume (meter): full volume based on meter reads - Transition Volume (RV): full volume based on RV calculations - Schedule 3, 29e or SEES Exempt Customers: full volume - Vacant: based on the last Meter Advance - Temporary Disconnection: the volume for the period spanning a temporary disconnection is set to zero |
| Charge / pence | The charge displayed is the Volumetric Charge associated to each SPID that falls under the Service Element CMS for the LP (the sum for all SPIDs under that CMS). The charge is calculated as follows: - Transition Charge (meter and RV): the relevant % discount to the full charge is applied - Schedule 3, 29e: the relevant % discount to the full charge is applied - SEES (Exempt Customers): no charge, apply proportion of the SEES Credit to each of the Service Elements for this SPID - Vacant: zero charge for the period spanning the vacancy - Temporary Disconnection: zero charge for the period spanning the temporary disconnection |

Section 4 - Water Non-volumetric Charges

SPIDs subject to Transitional charges will appear twice; once for the physical based Chargeable Meter Size (CMS) and once for the Rateable Value based CMS.

| Service Elements | The column labelled Service Element provides the level of Reporting in Aggregate for each “Chargeable Meter Size” (CMS) that has been notified or derived. In addition, other types of Service Elements are also provided here for Miscellaneous Water Services (this includes Outside Taps and Field Troughs for both Farms and Crofts). Meter and Unmeasureable CMS are reported separately. Unmeasureable CMS includes all RV derived Supply Points (i.e. Unmeasureable and Transition Arrangement SPIDs). |
| --- | --- |
| Number of registered days | The Number of Registered Days describes how many days the LP has been registered to each SPID that is aggregated for the Service Element being reported. Regardless of whether the SPID is registered as Vacant, Temporary Disconnected or a Transition percentage is being applied. There are two counts for Transition SPIDs with meters, the physical based CMS and the RV based CMS. |
| Charge / pence | The charge displayed is the fixed charge associated to each SPID that falls under the Service Element (CMS or Miscellaneous Service type) for the LP (the sum for all SPIDs under that Service Element). The charge is calculated as follows: - Transition Charge (meter and RV): the relevant % discount to the full charge is applied - Schedule 3, 29e: the relevant % discount to the full charge is applied - SEES (Exempt Customers): no charge, apply proportion of the SEES credit to each of the Service Elements for this SPID - Vacant: zero charge for period they are vacant - Temporary Disconnection: fixed charges are unaffected by temporary disconnection |

Section 5 - Sewerage Volumetric Charges

These exclude the fixed charge based elements of sewerage meter based charges. SPIDs subject to Transitional charges will appear twice; once for the physical based Chargeable Meter Size (CMS) and once for the Rateable Value based CMS.

| Service Elements | The column labelled Service Element provides the level of Reporting in Aggregate for each Sewerage “Chargeable Meter Size” (CMS) that has been notified or derived. Meter and Unmeasureable CMS are reported separately. Unmeasureable CMS includes all RV derived Supply Points (i.e. Unmeasureable and Transition Arrangement SPIDs). |
| --- | --- |
| Number of registered days | The Number of Registered Days describes how many days the LP has been registered to each SPID that is aggregated for the Service Element being reported. Regardless of whether the SPID is registered as Vacant, Temporary Disconnected or a Transition percentage is being applied. There are two counts for Transition SPIDs with meters, the physical based CMS and the RV based CMS. The Meter associated to the SPID is counted even if the Return to Sewer allowance is zero and/or a Non Domestic Allowance exists for Trade Effluent. |
| Volume / m3 | The volume is either a) the volume associated to the meter or the RV (for Unmeasureable CMS) adjusted for the Non-Return to Sewer allowance or b) the Non Domestic Allowance (NDA) where the SPID has associated Trade Effluent Discharge Point(s). It is the sum of Daily Volumes for each day the LP is registered to each SPID summed up for that Service Element CMS. The volume figure is calculated as follows: - Transition Volume (meter): full volume based on meter reads - Transition Volume (RV): full volume based on RV calculations - Schedule 3, 29e or SEES Exempt Customers: full volume - Vacant: based on the last Meter Advance - Temporary Disconnection: the volume for the any period spanning a temporary disconnection is set to zero. |
| Charge / pence | The charge displayed is the charge associated to each SPID that falls under the Service Element CMS for the LP (the sum for all SPIDs under that CMS). The charge is calculated as follows: - Transition Charge (meter and RV): the relevant % discount is applied to the full charge - Schedule 3, 29e: the relevant % discount is applied to the full charge - SEES (Exempt Customers): no charge, apply proportion of the SEES Credit to each of the Service Elements for this SPID - Vacant: zero charge is applied for period of the vacancy - Temporary Disconnection: zero charge is applied for period temporary disconnection |

Section 6 - Sewerage Non-volumetric Charges

SPIDs subject to Transitional charges will appear twice; once for the physical based Chargeable Meter Size (CMS) and once for the Rateable Value based CMS.

| Service Elements | The column labelled Service Element provides the level of Reporting in Aggregate for each Sewerage “Chargeable Meter Size” (CMS) that has been notified or derived. In addition, other types of Service Elements are also provided here for Miscellaneous Sewerage Services (this Property Drainage, Road Drainage and Caravans). Meter and Unmeasureable CMS are reported separately. Unmeasureable CMS includes all RV derived Supply Points (i.e. Unmeasureable and Transition Arrangement SPIDs). |
| --- | --- |
| Number of registered days | The Number of Registered Days describes how many days the LP has been registered to each SPID that is aggregated for the Service Element being reported. Regardless of whether the SPID is registered as Vacant, Temporary Disconnected or a Transition percentage is being applied. There are two Counts for Transition SPIDs with meters, the physical based CMS and the RV Based CMS. Note: Where the Non-Return to Sewerage Allowance (NRSA) is set to zero, the SPID will not be counted. However, where there is a Trade Effluent Non Domestic Allowance associated to a meter then registered days are counted because there will be a volume that over rides any zero NRSA on the meter |
| Charge / pence | The charge displayed is the charge associated to each SPID that falls under the Service Element (CMS or Miscellaneous service type) for the LP (the sum for all SPIDs under that Service Element). The charge is calculated as follows: - Transition Charge (meter and RV): the relevant % discount to the full charge is applied - Schedule 3, 29e: the relevant % discount to the full charge is applied - SEES (Exempt Customers): no charge, apply proportion of the SEES credit to each of the Service Elements for this SPID - Vacant: zero charge is applied for vacant periods - Temporary Disconnection: fixed charges are unaffected by temporary disconnection |

Section 7 - Trade Effluent Charges

| Service Elements | The column labelled Service Element provides Discharge Point (DPID) level reporting. |
| --- | --- |
| Number of registered days | The Number of Registered Days describes how many days the LP has been registered to each DPID for the Invoice Period. |
| Volume / m3 | The volume is sum the Daily Actual Volume Discharged (DAVD) for the period or the last DAVD used as an estimate where a TE Volume submission does not cover the period. |
| Charge / pence | The charge displayed is the charge associated to each DPID. The percentage discount will be applied to DPIDs with Schedule 3 arrangements on them. |